|  | Release Notes Spectrum Controls 1769sc-IF4IH DTM Software Version 1.0.0.5 |
| --- | --- |

|  | Topic; Page. About the DTM Software and Version Update; 1. Required System Components; 2. Known Anomalies; 3. Compatibility Issues; 3. Install DTM Library Software; 3. Update the DTM Catalogue; 4. Additional Resources; 4. |
| --- | --- |

| # About the DTM Software and Version Update | The Spectrum Controls 1769sc-IF4IH DTM Software is a Device Type Manager (DTM) for use with a CompactLogix system containing 1769sc-IF4IH (Highway Addressable Remote Transducer) I/O modules. The DTMs allow field device tools (FDTs), such as Endress+Hauser FieldCare or FactoryTalk AssetCentre, to communicate with HART devices through the CompactLogix and HART I/O modules. |
| --- | --- |

| # Required System Components | The following section describes the system and software required to install the Spectrum Controls DTM software. ## Operating System Use one of the following operating systems to run the DTM software: - Windows XP operating system with service pack 2 - Windows 7 (32 bit) operating system ## FDT Frame Applications Spectrum Controls DTMs were tested for use in the following FDT Frame applications: - Rockwell Automation FactoryTalk AssetCentre software - Endress+Hauser FieldCare software - Pepperl & Fuchs PACTware software - M&M Software fdtContainer software ## MSXML 4.0 required Microsoft MSXML 4.0 is required. ## Hard Disk Requirements You need these hard disk space requirements: - Approximately 10 MB during operation - Approximately 30 MB during installation ## Other System Resources For the DTM to execute properly, the following system resources must be available: - CPU of at least P3, 500 MHz, or equivalent - Working memory (RAM) of at least 256 MB ## Module Requirements The following are required for the DTM to communicate successfully with the module: - Firmware rev 2 or later. - RSLogix projects created using the module’s add-on profile v1.3 or later. |
| --- | --- |

| # Known Anomalies | - Attempting to connect to a CompactLogix on an incorrect IP Address will cause the FDT Frame Application to hang. - Attempting to connect to a CompactLogix that is unreachable (i.e. Ethernet cable disconnected) will cause the FDT Frame Application to hang. - HART Pass Though Query Requests greater than 38 bytes are not supported. |
| --- | --- |

| # Compatibility Issues | Projects created with v1.0.0.0 will only communicate with modules that have been configured using the 1769-Generic Profile in RSLogix. Projects created with FieldCare prior to version 2.07 will have issues closing the project, where FieldCare may report an error. |
| --- | --- |

| # Install DTM Library Software | Complete the following tasks to install the DTM Library software. ## Use the Setup Wizard to Install the Software Complete the following steps to install the DTM software. - Double-click the ‘SC 1769sc-IF4IH DTM 1.0.0.5 Setup.exe’ file. The setup wizard opens. - Click Next to continue the installation. - Accept the terms of the license agreement and click Next. - The setup wizard should begin the installation. The setup wizard indicates that the installation is complete. |
| --- | --- |

| # Update the DTM Catalog | After you have installed the DTM Library software, you must update the DTM catalog in the Endress+Hauser FieldCare, FactoryTalk AssetCentre software. ## Update with FieldCare Software Complete these steps to update the DTM catalog. - Open the FieldCare software. - From the menu bar, select DTM Catalog > Update. - Click Update. The software searches for and displays the DTMs. - Select the newly-installed DTMs and click Move. The selected DTMs are added to the catalog. - Click OK to save your changes and exit. ## Update with FactoryTalk AssetCentre Software Complete these steps to update the DTM catalog. - Open the FactoryTalk AssetCentre Software. - From the menu bar, select Tools > DTM Catalog. - Click Scan. The software searches for and installs the new DTMs. |
| --- | --- |

| # Additional Resources | These documents contain additional information concerning related Spectrum Controls products. You can view or download publications at http://www.spectrumcontrols.com/. Resource; Description. Compact I/O Isolated HART Analog Module, User Manual 0300215-02 Rev. B; This manual describes how to install, configure, and troubleshoot your CompactLogix HART analog I/O modules.. RSLogix add-on profile.; This install allows one to add the device in the RSLogix IO tree and generate module tags.. |
| --- | --- |