Complete InView AOI User Configuration Instructions 09/23/08

System Compatibility

The add-on instruction file is compatible with:

RSLogix5000 V17 or later

NOTE: These instructions are for InView displays communicating over Ethernet/IP only.

2706-P22R

2706-P42

2706-P43

2706-P44

2706-P72C

2706-P74C

2706-P92C

2706-P94C

Overview Section

This AOI creates the entire InView string that downloads and triggers a message for display.  The message is also now stored in the InView for future use.  This AOI allows the user to create his own messages in the ladder logic rather than using InView Messaging Software.  The user has the ability to change message attributes such as text color, character font, text position, etc.  However, the user does not have the ability to insert variables into the message – use InView Communication module to create messages containing variables or other objects such as date & time.  More detail is provided later in this document.  A sample ladder logic file is included with this AOI zip package.

General Setup

The following instructions outline the steps, in order, that are necessary to successfully download & trigger messages on an InView message display.

Step One – Configure the InView Communication Module

Open your web browser and enter the IP address of the InView comms module (Default IP is 192.168.1.100).

- Log in to the inview comms module.  User name is “admin”, password is “spectrum”.

- Select Industrial Network.

- Configure your PLC network settings.

- Click the “Test Device Connection” button to confirm communication.

- Click the “submit” button to accept the new settings.

The following step specifies the tags within the controller that will contain the data and trigger values.  Enter both the Message & Variable information.

- Enter ‘Msg_Data[0]’ for the Message Trigger Address and ‘Msg_Data[1]’ for the Message Data Address.  Enter ‘Var_Data[0]’ for the Variable Trigger Address and ‘Var_Data[1]’ for the Variable Data Address.  Enter 86 for the message array size and 20  for the variable array size.

Note:

Trigger Address: this is the address that the InView communication module is monitoring to know when to output the data located at the Data Address.

Data Address: This is the address that contains the InView protocol for either triggering a message or updating a variable.

- Click the “submit” button to save your changes.

The next step describes how to download your settings to the InView Comms module.  The process describing how to add messages to the InView comms module has been omitted.  For details on how to add messages, please refer to the InView Comms Software User Guide.

- Clcik on the “Messages” button.

- Select your message group and then click the “download” button.

Step Two – Logix Configuration

- Copy “Complete InView AOI Files” folder and contents onto your development P.C..

- Open the existing ladder logic file with RSLogix5000 v17 or newer.

- Import all the add-on instructions – file extension is L5X.

- Create Controller tags and assign the corresponding data types for the AOI tags shown below.  The controller tags you create will be used to pass the data to the AOI.  See the AOI Sample Code section for an example.

**Note** Msg_Data is a SINT of size 87 (this allows for a message size of 50 bytes or characters & the message attributes).  The controller tag name you create for this AOI tag must be the same name as the Message Data Address name that was assigned in Step 6 on page 3.

Advanced Message Download AOI Sample Code

Below is a screen shot of a sample ladder program that is included with the Complete InView AOI files.  The AOI will execute each time New_message receives new text to display.  As an example, to download & trigger a message and store it as message number 5 on an InView with a serial address of 20, the main ladder would manipulate the following tags:

New_message = This is the message text to display

Display_Address = 14 (hex value for decimal 20)

Dec_msg_number = 5

The rest of the attributes can remain at 0.  For test purposes, one can just manually enter these values directly into the appropriate controller tags.

The AOI will write the InView string to Msg_Data[1] and then change the state of  Msg_Data[0].  This will cause the string to be sent out of the serial port of the InView communication module.  See Step One in this document for information regarding configuring the InView communication module.

See the following pages for more info regarding the AOI inputs.

This is the add-on

instruction

The following is a list of the AOI inputs and a range of values for each input.

- Display_Address – hex value of display address from 1 to 254 (FE).  Use 00 or FF depending on which InView is used for a display address of 255, which signifies a message broadcast.  Two decimal places are required – display address of 1 would be 01.

- Dec_msg_number – decimal value of the desired message number for storage & future reuse.  The values range from 1 to 4000 based on the partition size of the InView.  The background message is 4095.  In order to be able to store 4000 messages, the InView partition would be 50 – message size of 50 characters or bytes.  A partition size of 100 would result in only 2000 messages being able to be stored.  If the messages are intended to be stored in the controller, a message number of 1 can be used for all messages to display.

- Position – position on the InView where the message is to appear.  The decimal values range from 0 to 3.

  - Middle = 0

  - Top = 1

  - Bottom = 2

  - Fill = 3

- Mode – determines whether the message rotates or not.  The decimal values are either 0 or 1.

  - Rotate left = 0

  - Hold = 1

- Justification – determines whether the message is centered or left justified.  The decimal values are either 0 or 1.

  - Left = 0

  - Center = 1

- Font – determines the character font.  The decimal values range from 0 to 7.

  - 5 high standard = 0

  - 7 high standard = 1

  - 7 high fancy = 2

  - 10 high standard = 3

  - 16 high fancy = 4

  - 16 high standard = 5

  - 24/32 fancy = 6

  - 24/32 high = 7

- Color – determines the character color.  The decimal value ranges from 0 to 2.

  - Red = 0

  - Green = 1

  - Yellow = 2

- Flash – determines whether the text flashes or not.  The decimal values are either 0 or 1.

  - Flash off = 0

  - Flash on = 1

- New_message – this contains the text to display on the InView.  If using a Sint of 87 for the output to the InView (this is what is used in the sample code – Msg_Data – and it includes the New_message text as well as the message attributes), the message text can be as large as 50 characters.  This also means that the InView would need to be set for a partition of 100 characters.

Message Trigger AOI Sample Code

Below is a screen shot of a sample ladder program that has been included with the Complete InView AOI files.  The AOI will execute each time New_msg_number receives a new message number to display.  As an example, to broadcast trigger message number 750, the main ladder would manipulate the following tags:

New_msg_number = 750

Display_Address2 = 255

For test purposes, one can just manually enter these values directly into the appropriate controller tags.

The AOI will write the InView string to Msg_Data[1] and then change the state of  Msg_Data[0].  This will cause the string to be sent out of the serial port of the InView communication module.  See Step One in this document for information regarding configuring the InView communication module.

This is the add-on

instruction

Variable Trigger AOI Sample Code

Below is a screen shot of a sample ladder program that has been included with the Complete InView AOI files.  The AOI will execute each time New_var_data receives new variable data to display.  As an example, to update variable number 90 with variable data 425, on any InView (broadcast message - serial address of 255), the main ladder would manipulate the following tags:

New_var_data = 425

Var_number = 90

Display_Address2 = 255

For test purposes, one can just manually enter these values directly into the appropriate controller tags.  Also, in order to view the variable data, the message containing the variable number would need to be triggered.

The AOI will write the InView string to Var_Data[1] and then change the state of  Var_Data[0].  This will cause the string to be sent out of the serial port of the InView communication module.  See Step One in this document for information regarding configuring the InView communication module.

This is the add-on

instruction

Alphanumeric Variable AOI Sample Code

Below is a screen shot of a sample ladder program that has been included with the Complete InView AOI files.  The AOI will execute each time New_alpha_var_data receives new alphanumeric data to display.  As an example, to update alphanumeric variable number 25 with alphanumeric data WGM62570 on any InView (broadcast update), the main ladder would manipulate the following tags:

Alpha_var_number = 25

New_alpha_var_data = WGM62570

Display_Address2 = 255

For test purposes, one can just manually enter these values directly into the appropriate controller tags.  Also, in order to view the updated alphanumeric data, the message containing the alphanumeric variable number to be updated must be triggered.

The AOI will write the InView string to Var_Data[1] and then change the state of  Var_Data[0].  This will cause the string to be sent out of the serial port of the InView communication module.  See Step One in this document for information regarding configuring the InView communication module.

This is the add-on

instruction